HMB Choto

45285209

# CMPG 325

# Milestone 2
Milestone Implementation Report

Contents

Project overview	3

Key design constraints	3

Physical topology	3

Devices	4

Logical Topology and VLAN Design	4

VLAN allocation	5

IP Addressing Plan	6

Key Static Assignments:	6

Router-on-a-Stick Configuration	7

NAT and Internet access	8

Server Configuration	9

Access Control List Policy	9

Policy Summary	9

ACL configuration set on R1	10

Testing and Verification	12

Before ACL	12

Post ACL verification	14

NAT Verification	17

Routing Verification	17

Conclusion	17

# Project overview

North-West Asset Finance is a banking and finance client operating from a single Rustenburg site. The network prioritises departmental segmentation and controlled access to shared resources, while still allowing operational services such as printing to be shared efficiently across the business.

## Key design constraints

- Printer sharing must cross departments.

- File sharing must not cross departments.

- Guest Wi-Fi must be isolated from all internal resources (Change Request CR3).

- Assigned addressing block: 192.168.14.0/24.

# Physical topology

The physical design uses a collapsed-core architecture: a single Layer-3 edge router (R1 / NWAF-RTR01) performs inter-VLAN routing (router-on-a-stick) and hosts the ACL policy; a core/distribution switch (SW-CORE) trunks VLANs down to per-department access switches; a dedicated server switch (SW-SERVER) hosts the File Server and Print Server; and a wireless access point (AP-GUEST) serves the isolated guest network.

## Devices

| Device | Role | Model |
| --- | --- | --- |
| ISP Router | Simulated Internet | 2911 |
| R1 (NWAF-RTR01) | Edge router, ROAS, ACLs, NAT | ISR 4331 |
| SW-CORE | Core/distribution switch | 2960-24TT |
| SW-ADMIN | Access switch — VLAN 10 | 2960-24TT |
| SW-LOANS | Access switch — VLAN 20 | 2960-24TT |
| SW-CLIENTS | Access switch — VLAN 30 | 2960-24TT |
| SW-SERVER | Access switch — VLAN 40 | 2960-24TT |
| AP-GUEST | Wireless access point | Access Point |
| File Server | Centralised file storage | Server-PT |
| Print Server | Centralised print services | Server-PT |

# Logical Topology and VLAN Design

Each department, the server farm, and guest Wi-Fi are placed on separate VLANs, each mapped to its own subnet carved from the assigned 192.168.14.0/24 block using VLSM.

VLAN configuration was verified directly on the switches. The interface status on SW-SERVER confirms the File Server and Print Server ports are connected and correctly placed in VLAN 40, with the uplink to SW-CORE operating as a trunk:

show interfaces status — SW-SERVER (Fa0/1, Fa0/2 in VLAN 40; Gi0/1 trunking to SW-CORE)

The VLAN database on SW-SERVER confirms all five VLANs (10, 20, 30, 40, 99) are active:

show vlan brief — SW-SERVER

Confirmation the trunk carries all VLANs at the distribution layer:

show vlan brief — SW-CORE

## VLAN allocation

| VLAN | Department / Purpose | Subnet | Gateway |
| --- | --- | --- | --- |
| 10 | Administration | 192.168.14.0/27 | 192.168.14.1 |
| 20 | Loans | 192.168.14.32/27 | 192.168.14.33 |
| 30 | Client Services | 192.168.14.64/27 | 192.168.14.65 |
| 40 | Servers (File + Print) | 192.168.14.96/28 | 192.168.14.97 |
| 99 | Guest Wi-Fi | 192.168.14.112/28 | 192.168.14.113 |
| — | Reserved (future) | 192.168.14.128/25 | — |

# IP Addressing Plan

Subnetting was performed with VLSM against the assigned block 192.168.14.0/24, sized to each VLAN's expected host count while leaving a reserved block for future growth.

| VLAN | Network | Mask (CIDR) | Usable Range | Gateway | Hosts |
| --- | --- | --- | --- | --- | --- |
| 10 | 192.168.14.0 | /27 | .1 – .30 | 192.168.14.1 | 30 |
| 20 | 192.168.14.32 | /27 | .33 – .62 | 192.168.14.33 | 30 |
| 30 | 192.168.14.64 | /27 | .65 – .94 | 192.168.14.65 | 30 |
| 40 | 192.168.14.96 | /28 | .97 – .110 | 192.168.14.97 | 14 |
| 99 | 192.168.14.112 | /28 | .113 – .126 | 192.168.14.113 | 14 |
| — | 192.168.14.128 | /25 | .129 – .254 | — | 126 (reserved) |

## Key Static Assignments:

| VLAN | Network | Mask (CIDR) | Usable Range | Gateway | Hosts |
| --- | --- | --- | --- | --- | --- |
| 10 | 192.168.14.0 | /27 | .1 – .30 | 192.168.14.1 | 30 |
| 20 | 192.168.14.32 | /27 | .33 – .62 | 192.168.14.33 | 30 |
| 30 | 192.168.14.64 | /27 | .65 – .94 | 192.168.14.65 | 30 |
| 40 | 192.168.14.96 | /28 | .97 – .110 | 192.168.14.97 | 14 |
| 99 | 192.168.14.112 | /28 | .113 – .126 | 192.168.14.113 | 14 |
| — | 192.168.14.128 | /25 | .129 – .254 | — | 126 (reserved) |

# Router-on-a-Stick Configuration

R1 performs inter-VLAN routing using a single physical LAN interface (GigabitEthernet0/0/1) subdivided into 802.1Q sub-interfaces — one per VLAN.

R1 sub-interface configuration:

interface GigabitEthernet0/0/1
 no shutdown
!
interface GigabitEthernet0/0/1.10
 encapsulation dot1Q 10
 ip address 192.168.14.1 255.255.255.224
!
interface GigabitEthernet0/0/1.20
 encapsulation dot1Q 20
 ip address 192.168.14.33 255.255.255.224
!
interface GigabitEthernet0/0/1.30
 encapsulation dot1Q 30
 ip address 192.168.14.65 255.255.255.224
!
interface GigabitEthernet0/0/1.40
 encapsulation dot1Q 40
 ip address 192.168.14.97 255.255.255.240
!
interface GigabitEthernet0/0/1.99
 encapsulation dot1Q 99
 ip address 192.168.14.113 255.255.255.240

Verification confirms all five sub-interfaces are up/up, alongside the WAN interface and the default route to the simulated ISP:

show ip interface brief — R1, captured alongside the NAT and default-route configuration session

# NAT and Internet access

NAT overload (PAT) is configured on R1 so all internal VLANs can reach the Internet via the WAN interface. A default static route points to the simulated ISP.

interface GigabitEthernet0/0/0
 ip address 10.0.0.2 255.255.255.252
 ip nat outside
 no shutdown
!
ip route 0.0.0.0 0.0.0.0 10.0.0.1
!
access-list 1 permit 192.168.14.0 0.0.0.255
ip nat inside source list 1 interface GigabitEthernet0/0/0 overload

show ip nat statistics — R1, immediately after NAT configuration

Immediately after configuration, NAT statistics confirm the inside/outside interfaces are correctly registered, zero hits at this point, since no traffic had yet been generated:

The routing table confirms all five LAN subnets as directly connected, plus the WAN link and the default route via 10.0.0.1:

show ip route — R1, baseline routing table

# Server Configuration

Both servers reside in VLAN 40 on the server switch (SW-SERVER), configured with static IPs in the 192.168.14.96/28 subnet.

| Server | IP Address | Subnet Mask | Default Gateway |
| --- | --- | --- | --- |
| File Server | 192.168.14.98 | 255.255.255.240 | 192.168.14.97 |
| Print Server | 192.168.14.99 | 255.255.255.240 | 192.168.14.97 |

# Access Control List Policy

ACLs enforce the client's sharing policy at R1, applied inbound on each VLAN sub-interface so traffic is filtered as it enters the router from the LAN side.

## Policy Summary

| Rule | Action | Purpose |
| --- | --- | --- |
| Admin VLAN → File Server | Permit | Own department's file share |
| Admin VLAN → Print Server | Permit | Cross-department printing |
| Loans VLAN → File Server | Deny | Cross-department file access blocked |
| Loans VLAN → Print Server | Permit | Cross-department printing allowed |
| Clients VLAN → File Server | Deny | Cross-department file access blocked |
| Clients VLAN → Print Server | Permit | Cross-department printing allowed |
| Any internal VLAN → Internet | Permit (NAT) | General business access |
| Guest VLAN → All internal VLANs | Deny | Guest isolation (CR3) |
| Guest VLAN → Internet | Permit (NAT) | Guest internet only |
| (Implicit deny all) | Deny | Standard IOS behaviour |

## ACL configuration set on R1

The access lists were defined and bound to each VLAN sub-interface directly on the router console:

ACL definition and application — R1 console session

show ip access-lists confirms the four extended ACLs are active, with non-zero match counters from the verification pings performed in Testing and verification Section:

show ip access-lists — R1, post-testing match counters

# Testing and Verification

## Before ACL

Before ACLs were applied, Admin and Loans could both reach the File Server, Print Server, and the Internet. The first ping attempt to a newly-contacted host occasionally timed out once before succeeding — a normal ARP-resolution delay in Packet Tracer, not a connectivity fault.

Admin PC — baseline pings to .97 (gateway), .98 (File Server), .99 (Print Server)

Loans PC — baseline pings to .98 (File Server) and .99 (Print Server), both succeeding pre-ACL

## Post ACL verification

After the ACLs were applied, Admin PC retains full access to its own File Server, the Print Server, and the Internet, consistent with ADMIN_ACL:

Admin PC post-ACL — .98 and .99 succeed, 8.8.8.8 succeeds

Loans PC is now blocked from pinging the File Server, while printing and Internet access remain permitted — the before/after contrast in a single capture is evidence the LOANS_ACL deny rule is active for ICMP. Note the caveat in Section 8: this confirms ICMP is blocked, not yet that real file-sharing (TCP/445, 139) is blocked — see the fix required there.

Loans PC — pre-ACL success to .98, then post-ACL: .98 blocked, .99 and Internet still succeed

The Guest laptop is blocked from every internal host it was tested against, while Internet access remains available, confirming guest isolation (CR3):

Guest laptop post-ACL — .98 and .99 blocked, 8.8.8.8 succeeds

## NAT Verification

Translation entries confirm internal hosts are being translated to the WAN address when reaching the Internet:

show ip nat translations — R1, active ICMP translations during testing

NAT statistics after the full test pass show a healthy hit count across the inside interfaces:

show ip nat statistics — R1, post-testing (26 hits)

## Routing Verification

The routing table remains stable after ACL deployment — ACLs filter traffic, they do not alter routing — with all five LAN subnets, the WAN link, and the default route all present:

show ip route — R1, post-ACL

# Conclusion

The design for North-West Asset Finance has been implemented and tested in Cisco Packet Tracer. Key deliverables achieved:

- Segmentation: five VLANs (Admin, Loans, Clients, Servers, Guest) with VLSM addressing from 192.168.14.0/24.

- Scalability: a reserved /25 block retained for future growth.

- Inter-VLAN routing: router-on-a-stick implemented at R1 with 802.1Q sub-interfaces, all confirmed up/up.

- Internet access: NAT overload configured and verified, with active translations and a default route to the ISP.

- ACL policy: cross-department printing and guest isolation are both confirmed working. Cross-department file-sharing restriction is confirmed for ICMP

- Guest isolation (CR3): Guest Wi-Fi is fully isolated from all internal VLANs, with Internet-only access confirmed.